Figure 2: y-direction magnitude by time plot and corresponding Fourier transform

Figure 1: x-direction magnitude by time plot and corresponding Fourier transform

Figure 3: z-direction magnitude by time plot and corresponding Fourier transform

- import numpy as np

- import scipy as sy

- import scipy.fftpack as syfp

- import matplotlib.pyplot as plt

- array = np.loadtxt("D:\\Desktop\\accelxyz.csv", delimiter=',')

- column_num = 2  #0 means x-axis, 1 means y-axis and 2 means z-axis

- length = len(array[:,column_num]) #Number of data points

- x = sy.linspace(0.005, length*0.008, num=length) #Return evenly spaced numbers as x-axis values

- yf = syfp.fft(array[:,column_num]) #Discrete Fourier transform of array

- f = syfp.fftfreq(length, np.mean(np.diff(x))) #Return the Discrete Fourier Transform sample frequencies

- plt.subplot(211) #Create signal magnitude by time plot

- plt.plot(x, array[:,column_num]) #Plot values

- plt.subplot(212) #Create magnitude by frequency plot

- plt.plot(abs(f), abs(yf)) #Plot values

- plt.show()

Figure 6: Original and filtered sensor data in the z-direction along with Fourier transform of filtered sensor data

Figure 5: Original and filtered sensor data in the y-direction along with Fourier transform of filtered sensor data

Figure 4: Original and filtered sensor data in the x-direction along with Fourier transform of filtered sensor data

- import numpy as np

- import scipy as sy

- import scipy.fftpack as syfp

- import matplotlib.pyplot as plt

- from scipy.signal import butter, lfilter, freqz

- def butter_lowpass(cutoff, fs, order=3):

- nyq = 0.5 * fs

- normal_cutoff = cutoff / nyq

- b, a = butter(order, normal_cutoff, btype='low', analog=False)

- return b, a

- def butter_lowpass_filter(data, cutoff, fs, order=3):

- b, a = butter_lowpass(cutoff, fs, order=order)

- y = lfilter(b, a, data)

- return y

- array = np.loadtxt("D:\\Desktop\\accelxyz.csv", delimiter=',')

- column_num = 0 #0 means x-axis, 1 means y-axis and 2 means z-axis

- order = 3 # filter order

- fs = 125.0 # sample rate, Hz

- cutoff = 15 # cut-off frequency

- length = len(array[:,column_num]) #Number of data points

- x = sy.linspace(0.005, length*0.008, num=length) #Return evenly spaced numbers as x-axis values

- y = butter_lowpass_filter(array[:,column_num], cutoff, fs, order)

- plt.subplot(2, 1, 1)

- plt.plot(x, array[:,column_num], 'b-', linewidth=1, label='data')

- plt.plot(x, y, 'g-', linewidth=1, label='filtered data')

- plt.xlabel('Time [sec]')

- plt.ylabel('Magnitude')

- plt.grid()

- plt.legend()

- yf = syfp.fft(y) #Discrete Fourier transform of array

- f = syfp.fftfreq(length, np.mean(np.diff(x))) #Return the Discrete Fourier Transform sample frequencies

- plt.subplot(212) #Create magnitude by frequency plot

- plt.plot(abs(f), abs(yf), 'r-', linewidth=1) #Plot values

- plt.xlabel('Frequency [Hz]')

- plt.ylabel('Magnitude')

- plt.grid()

- plt.show()

Figure 7: Closer look at raw data and filtered data